Flood Susceptibility Mapping — Threshold Optimisation & Validation Report

Slovenia | EPSG:3794 | 12.5 m | 10x sample size (500k/class) | RF · XGBoost · CatBoost · DNN · Ensemble

1. Optimal Thresholds

The CSI-optimal threshold minimises false alarms and misses simultaneously, making it the preferred choice for flood hazard mapping. Youden's J maximises the sum of sensitivity and specificity. Optimal threshold points are marked on the ROC curves.

| Model | CSI_threshold | CSI_value | Youden_threshold | Youden_value |
| --- | --- | --- | --- | --- |
| RF | 0.5172 | 0.9186 | 0.5616 | 0.9142 |
| XGBoost | 0.5369 | 0.9406 | 0.5468 | 0.9382 |
| CatBoost | 0.5222 | 0.9255 | 0.5222 | 0.9217 |
| DNN | 0.532 | 0.9085 | 0.532 | 0.902 |
| Ensemble_mean | 0.5566 | 0.928 | 0.5714 | 0.9245 |

2. Validation Metrics (Test Set)

Metrics computed on 199,495 test samples (10x split, held out from training) at CSI-optimal thresholds. AUC is threshold-independent; all other metrics use the optimal threshold.

| Model | Threshold | AUC | CSI | F1 | Accuracy | Kappa | Sensitivity | Specificity | Precision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XGBoost | 0.5369 | 0.9941 | 0.9406 | 0.9694 | 0.9691 | 0.9381 | 0.9824 | 0.9557 | 0.9567 |
| Ensemble_mean | 0.5566 | 0.9921 | 0.928 | 0.9627 | 0.9622 | 0.9244 | 0.9765 | 0.9479 | 0.9492 |
| CatBoost | 0.5222 | 0.991 | 0.9255 | 0.9613 | 0.9608 | 0.9216 | 0.9759 | 0.9458 | 0.9472 |
| RF | 0.5172 | 0.9903 | 0.9186 | 0.9576 | 0.9569 | 0.9138 | 0.9755 | 0.9383 | 0.9403 |
| DNN | 0.532 | 0.986 | 0.9085 | 0.952 | 0.951 | 0.9019 | 0.9752 | 0.9268 | 0.9299 |

3. Confusion Matrix Summary

XGBoost  TP=97826  TN=95495  FP=4425  FN=1749  (Threshold = 0.5369)

Ensemble_mean  TP=97233  TN=94719  FP=5201  FN=2342  (Threshold = 0.5566)

CatBoost  TP=97174  TN=94502  FP=5418  FN=2401  (Threshold = 0.5222)

RF  TP=97139  TN=93753  FP=6167  FN=2436  (Threshold = 0.5172)

DNN  TP=97107  TN=92603  FP=7317  FN=2468  (Threshold = 0.532)

4. Susceptibility Classification (5-Class Maps)

Five flood susceptibility classes were derived by quantile classification (equal-frequency) of the probability values over all valid pixels within Slovenia. Each class contains ~20% of the valid area.

| Model | Very Low (<q20) | Low (q20–q40) | Moderate (q40–q60) | High (q60–q80) | Very High (>q80) |
| --- | --- | --- | --- | --- | --- |
| RF | <0.0000 | 0.0000–0.0010 | 0.0010–0.0106 | 0.0106–0.1407 | >0.1407 |
| XGBoost | <0.0000 | 0.0000–0.0000 | 0.0000–0.0007 | 0.0007–0.0322 | >0.0322 |
| CatBoost | <0.0000 | 0.0000–0.0002 | 0.0002–0.0029 | 0.0029–0.0733 | >0.0733 |
| DNN | <0.0001 | 0.0001–0.0006 | 0.0006–0.0036 | 0.0036–0.0998 | >0.0998 |
| Ensemble_mean | <0.0001 | 0.0001–0.0006 | 0.0006–0.0053 | 0.0053–0.0932 | >0.0932 |

5. Output Files

| Folder | File | Description |
| --- | --- | --- |
| classified_maps/ | FSM_{model}_binary.tif | Binary flood map (0/1) at CSI-optimal threshold — Int16 |
| classified_maps/ | FSM_{model}_5class.tif | 5-class susceptibility map (1=Very Low … 5=Very High) — Int16 |
| validation/ | validation_metrics.csv | Full metrics for all models |
| validation/ | threshold_summary.csv | CSI & Youden optimal thresholds per model |
| validation/ | roc_validation.png | ROC curves with optimal threshold points marked |
| validation/ | confusion_matrices.png | Confusion matrix grids for all models |
| validation/ | csi_threshold_curves.png | CSI vs threshold curves per model |
| validation/ | metrics_comparison.png | Bar chart comparing all metrics across models |

Generated: 2026-05-25  |  Project: FSM Slovenia  |  Models: 10x (500k/class)