# Slovenia Flood Susceptibility Mapping — Analysis Report

This report summarises the results of Items 3–6 of the FSM analysis: class area statistics, frequency ratio analysis, comparison with national flood hazard zones, and spatial agreement across models.

# 1. Susceptibility Class Area Statistics (Item 3)

For each of the five models, pixels were counted per susceptibility class (1–5), converted to km², and expressed as a percentage of the total valid Slovenia area. The cross-tabulation column 'Pct_Flooded' shows what percentage of pixels in each class are historically flooded (from the Flooded_NoFlood reference raster).

Table 1: Class area statistics for all models

| Model | Class | Class_Label | Pixels | Area_km2 | Pct_Total | Pct_Flooded |
| --- | --- | --- | --- | --- | --- | --- |
| RF | 1 | Very Low | 25871433 | 4042.41 | 19.96 | 0.0 |
| RF | 2 | Low | 25932814 | 4052.0 | 20.01 | 0.0 |
| RF | 3 | Moderate | 25959647 | 4056.19 | 20.03 | 0.002 |
| RF | 4 | High | 25952851 | 4055.13 | 20.02 | 0.073 |
| RF | 5 | Very High | 25906069 | 4047.82 | 19.99 | 28.08 |
| XGBoost | 1 | Very Low | 25912509 | 4048.83 | 19.99 | 0.0 |
| XGBoost | 2 | Low | 25929422 | 4051.47 | 20.0 | 0.0 |
| XGBoost | 3 | Moderate | 25951374 | 4054.9 | 20.02 | 0.0 |
| XGBoost | 4 | High | 25896850 | 4046.38 | 19.98 | 0.01 |
| XGBoost | 5 | Very High | 25932659 | 4051.98 | 20.01 | 28.116 |
| CatBoost | 1 | Very Low | 25925520 | 4050.86 | 20.0 | 0.0 |
| CatBoost | 2 | Low | 25904126 | 4047.52 | 19.98 | 0.0 |
| CatBoost | 3 | Moderate | 25929126 | 4051.43 | 20.0 | 0.001 |
| CatBoost | 4 | High | 25944382 | 4053.81 | 20.02 | 0.04 |
| CatBoost | 5 | Very High | 25919660 | 4049.95 | 20.0 | 28.099 |
| Ensemble_mean | 1 | Very Low | 25890777 | 4045.43 | 19.97 | 0.0 |
| Ensemble_mean | 2 | Low | 25910989 | 4048.59 | 19.99 | 0.0 |
| Ensemble_mean | 3 | Moderate | 25993449 | 4061.48 | 20.05 | 0.001 |
| Ensemble_mean | 4 | High | 25897494 | 4046.48 | 19.98 | 0.03 |
| Ensemble_mean | 5 | Very High | 25930105 | 4051.58 | 20.0 | 28.099 |
| DNN | 1 | Very Low | 25923045 | 4050.48 | 20.0 | 0.0 |
| DNN | 2 | Low | 25900062 | 4046.88 | 19.98 | 0.0 |
| DNN | 3 | Moderate | 25945982 | 4054.06 | 20.02 | 0.002 |
| DNN | 4 | High | 25940833 | 4053.26 | 20.01 | 0.08 |
| DNN | 5 | Very High | 25912892 | 4048.89 | 19.99 | 28.066 |

# 2. Frequency Ratio Analysis (Item 4)

Frequency Ratio (FR) was computed for each conditioning factor at 1/10 spatial scale (~1.3 M pixels). Continuous factors were binned into 10 quantile classes; categorical factors (Hgk_grouped, Pedoloska_grouped, lito_poly_250_d96_grouped) used their natural unique values. FR > 1 indicates flood-prone classes; FR < 1 indicates relatively safe classes.

Table 2: Frequency Ratio values (first 60 rows; full data in frequency_ratio.csv)

| Feature | Bin | Bin_Label | Total_Px | Flood_Px | FR |
| --- | --- | --- | --- | --- | --- |
| Distance_to_River | 0 | 0.00–77.49 | 129010 | 22439 | 3.0823 |
| Distance_to_River | 1 | 77.49–158.29 | 130012 | 14011 | 1.9098 |
| Distance_to_River | 2 | 158.29–242.95 | 129673 | 9788 | 1.3376 |
| Distance_to_River | 3 | 242.95–330.42 | 129674 | 7547 | 1.0314 |
| Distance_to_River | 4 | 330.42–423.14 | 129637 | 5715 | 0.7812 |
| Distance_to_River | 5 | 423.14–524.41 | 129561 | 4339 | 0.5935 |
| Distance_to_River | 6 | 524.41–639.12 | 129640 | 3188 | 0.4358 |
| Distance_to_River | 7 | 639.12–781.15 | 129608 | 2277 | 0.3113 |
| Distance_to_River | 8 | 781.15–987.39 | 129601 | 1631 | 0.223 |
| Distance_to_River | 9 | 987.39–3317.91 | 129607 | 2211 | 0.3023 |
| Distance_to_Road | 0 | 0.00–18.26 | 128102 | 5726 | 0.7921 |
| Distance_to_Road | 1 | 18.26–36.49 | 128662 | 6220 | 0.8567 |
| Distance_to_Road | 2 | 36.49–59.79 | 131330 | 6569 | 0.8864 |
| Distance_to_Road | 3 | 59.79–87.31 | 129956 | 6870 | 0.9368 |
| Distance_to_Road | 4 | 87.31–118.59 | 129295 | 6688 | 0.9167 |
| Distance_to_Road | 5 | 118.59–156.09 | 129894 | 6833 | 0.9322 |
| Distance_to_Road | 6 | 156.09–203.97 | 129477 | 7010 | 0.9594 |
| Distance_to_Road | 7 | 203.97–271.42 | 129984 | 7492 | 1.0214 |
| Distance_to_Road | 8 | 271.42–392.59 | 129648 | 8304 | 1.135 |
| Distance_to_Road | 9 | 392.59–2269.63 | 129595 | 11183 | 1.5292 |
| Hgk_grouped | 1 | 1 | 42761 | 544 | 0.2254 |
| Hgk_grouped | 2 | 2 | 171500 | 43826 | 4.5286 |
| Hgk_grouped | 3 | 3 | 163792 | 1155 | 0.125 |
| Hgk_grouped | 4 | 4 | 10050 | 212 | 0.3738 |
| Hgk_grouped | 5 | 5 | 65021 | 18019 | 4.911 |
| Hgk_grouped | 6 | 6 | 205836 | 1391 | 0.1198 |
| Hgk_grouped | 7 | 7 | 191460 | 4727 | 0.4375 |
| Hgk_grouped | 8 | 8 | 251553 | 1792 | 0.1262 |
| Hgk_grouped | 9 | 9 | 1604 | 437 | 4.828 |
| Hgk_grouped | 10 | 10 | 192523 | 1010 | 0.093 |
| LULC | 1 | 1 | 114214 | 24533 | 3.8065 |
| LULC | 2 | 2 | 32985 | 1001 | 0.5378 |
| LULC | 3 | 3 | 241868 | 21676 | 1.5882 |
| LULC | 4 | 4 | 810619 | 13626 | 0.2979 |
| LULC | 5 | 5 | 67363 | 5791 | 1.5234 |
| LULC | 6 | 6 | 9296 | 6476 | 12.3454 |
| LULC | 7 | 7 | 19716 | 39 | 0.0351 |
| Pedoloska_grouped | 0 | 0 | 36061 | 4997 | 2.4556 |
| Pedoloska_grouped | 1 | 1 | 2684 | 5 | 0.033 |
| Pedoloska_grouped | 2 | 2 | 374453 | 1288 | 0.061 |
| Pedoloska_grouped | 3 | 3 | 154710 | 812 | 0.093 |
| Pedoloska_grouped | 4 | 4 | 27365 | 46 | 0.0298 |
| Pedoloska_grouped | 5 | 5 | 213014 | 4755 | 0.3956 |
| Pedoloska_grouped | 6 | 6 | 268132 | 1942 | 0.1283 |
| Pedoloska_grouped | 7 | 7 | 30944 | 220 | 0.126 |
| Pedoloska_grouped | 8 | 8 | 2644 | 0 | 0.0 |
| Pedoloska_grouped | 9 | 9 | 65045 | 27497 | 7.4914 |
| Pedoloska_grouped | 10 | 10 | 12390 | 14 | 0.02 |
| Pedoloska_grouped | 11 | 11 | 48555 | 899 | 0.3281 |
| Pedoloska_grouped | 12 | 12 | 60262 | 30669 | 9.0188 |
| River_Density | 0 | 0.03–0.35 | 129603 | 3993 | 0.546 |
| River_Density | 1 | 0.35–0.42 | 129602 | 2332 | 0.3189 |
| River_Density | 2 | 0.42–0.46 | 129601 | 2280 | 0.3118 |
| River_Density | 3 | 0.46–0.50 | 129603 | 2267 | 0.31 |
| River_Density | 4 | 0.50–0.53 | 129602 | 2595 | 0.3548 |
| River_Density | 5 | 0.53–0.57 | 129603 | 3010 | 0.4116 |
| River_Density | 6 | 0.57–0.61 | 129602 | 4035 | 0.5517 |
| River_Density | 7 | 0.61–0.67 | 129602 | 6609 | 0.9037 |
| River_Density | 8 | 0.67–0.77 | 129602 | 14807 | 2.0246 |
| River_Density | 9 | 0.77–1.19 | 129603 | 31218 | 4.2686 |

# 3. National Flood Map Comparison (Item 5)

National flood zones (Frequent, Rare, Extreme Rare) were rasterized to the model grid and overlaid with the Ensemble_mean and XGBoost 5-class maps. Two metrics are reported: (a) % of each national zone that falls in model class 4 or 5, and (b) % of the Very High class (5) that overlaps each national zone.

Table 3: National flood zone comparison

| Model | Flood_Zone | Zone_Area_km2 | Overlap_Cls4_5_km2 | Pct_Zone_in_Cls4_5 | VeryHigh_Area_km2 | Pct_VeryHigh_in_Zone |
| --- | --- | --- | --- | --- | --- | --- |
| Ensemble_mean | Frequent | 97.09 | 97.09 | 100.0 | 4051.58 | 2.4 |
| Ensemble_mean | Rare | 464.88 | 464.88 | 100.0 | 4051.58 | 11.46 |
| Ensemble_mean | Extreme_Rare | 1038.86 | 1038.84 | 100.0 | 4051.58 | 25.62 |
| XGBoost | Frequent | 97.09 | 97.09 | 100.0 | 4051.98 | 2.4 |
| XGBoost | Rare | 464.88 | 464.88 | 100.0 | 4051.98 | 11.47 |
| XGBoost | Extreme_Rare | 1038.86 | 1038.86 | 100.0 | 4051.98 | 25.63 |

# 4. Spatial Agreement (Item 6)

The agreement map counts, for each pixel, how many of the five models classify it as Very High susceptibility (class 5). Values range from 0 (no model) to 5 (all models). A consensus map of pixels with ≥4 models agreeing on Very High is discussed below.

Table 4: Spatial agreement statistics

| Models_Agree_VeryHigh | Pixels | Area_km2 | Pct_Slovenia |
| --- | --- | --- | --- |
| 0.0 | 98650083.0 | 15414.08 | 76.1055 |
| 1.0 | 3488073.0 | 545.01 | 2.6909 |
| 2.0 | 1690320.0 | 264.11 | 1.304 |
| 3.0 | 1734075.0 | 270.95 | 1.3378 |
| 4.0 | 2770868.0 | 432.95 | 2.1376 |
| 5.0 | 21289395.0 | 3326.47 | 16.4241 |

Consensus Very High area (≥4 models agreeing): 18.56% of Slovenia.