# RABA to LULC Conversion

Derivation of the LULC conditioning factor from the Slovenian RABA land-use database (release 2026-06-30)

1. Purpose and rationale

RABA (Evidenca dejanske rabe kmetijskih in gozdnih zemljisc) delineates land-use polygons from orthophotos at sub-metre precision, with a minimum mapping unit of 1000 m² for agricultural land. The analysis grid is 12.5 m, i.e. 156.25 m² per cell.

Rasterising the polygons directly at 12.5 m applies a pixel-centre rule: any feature smaller than one cell either flips to a neighbouring class or vanishes entirely, and the resulting aliasing is noise a learner will fit. The conversion therefore burns the polygons at a 10x finer grid (1.25 m) and block-aggregates to per-class areal fractions, from which the categorical LULC factor is then taken as the modal class. The intermediate fraction raster is retained so the sub-cell composition remains available.

2. Input data

| Item | Value |
| --- | --- |
| Source layer | RABA_20260630.shp (release 2026-06-30) |
| Polygon features | 1,698,891 |
| Distinct RABA_ID codes | 25 (all mapped, none dropped) |
| Attribute burned | RABA_ID |
| Source CRS | EPSG:3794 (D96/TM, Slovene National Grid) |
| Reference grid | CgFs_Aligned/Flooded_NoFlood.tif |
| Output grid | 19909 x 12996 @ 12.5 m, EPSG:3794 |
| Intermediate burn grid | 1.25 m (10x oversample, 100 sub-cells per target cell) |

The source layer and the reference grid share EPSG:3794, so no datum transformation was applied. The D48/GK to D96/TM conversion, which is grid-based and would otherwise introduce metre-level regional distortion on a 12.5 m grid, was not required.

3. Processing chain

RABA_20260630.shp
        |
        |  raba_fractions.py  (burn at 1.25 m, block-aggregate 10x10)
        v
raba_frac_202606.tif    9 bands, float32  -  7 fractions + coverage + modal
        |
        |  extract modal band, nodata = 0, LZW
        v
CgFs_Aligned/LULC.tif   1 band, float32, codes 1-7

Processing is windowed over 78 x 51 tiles so the full fine grid never resides in memory. Runtime was approximately 12 minutes single-threaded; the intermediate fraction raster is 401 MB (deflate, predictor 3).

4. RABA_ID to LULC class mapping

Classes are grouped by runoff and infiltration behaviour rather than agronomy. Three assignments therefore differ from the thematic MKGP reading: greenhouses (1190) are grouped with built-up land because a glass or plastic roof is hydrologically impervious; overgrown agricultural area (1410) and trees and shrubs (1500) are grouped as woody because their canopy interception resembles forest; and uncultivated agricultural land (1600) is grouped as grassland, being typically rank grass rather than bare soil.

| RABA_ID | Official class name | LULC class | Polygons |
| --- | --- | --- | --- |
| 1100 | Arable land | arable | 214,613 |
| 1160 | Hop field | arable | 558 |
| 1180 | Permanent crops on arable land | arable | 1,341 |
| 1211 | Vineyard | permanent_crop | 41,208 |
| 1212 | Nursery | permanent_crop | 66 |
| 1221 | Intensive orchard | permanent_crop | 8,331 |
| 1222 | Extensive orchard | permanent_crop | 104,821 |
| 1230 | Olive grove | permanent_crop | 8,318 |
| 1240 | Other permanent crop | permanent_crop | 5,300 |
| 1300 | Permanent grassland | grassland | 313,678 |
| 1321 | Swampy meadow | grassland | 2,148 |
| 1600 | Uncultivated agricultural land | grassland | 159,667 |
| 1410 | Overgrown agricultural area | woody | 152,658 |
| 1420 | Forest plantation | woody | 924 |
| 1500 | Trees and shrubs | woody | 259,607 |
| 1800 | Forest trees on agricultural land | woody | 23,535 |
| 2000 | Forest | woody | 152,220 |
| 3000 | Built-up area and related surface | built_up | 194,092 |
| 1190 | Greenhouse | built_up | 6,043 |
| 4100 | Swamp | water_wetland | 12 |
| 4210 | Reed | water_wetland | 51 |
| 4220 | Other marshy area | water_wetland | 1,512 |
| 7000 | Water | water_wetland | 28,528 |
| 5000 | Dried open area with special vegetation | bare_open | 15,649 |
| 6000 | Open area with little or no vegetation | bare_open | 4,011 |
|  | Total |  | 1,698,891 |

Class names as published in RABA_Read_me.pdf.

5. LULC class distribution

Cell counts and areal share for each class in CgFs_Aligned/LULC.tif. The modal share is the proportion of cells whose dominant class is the given class; the fractional share is the mean areal fraction across all cells, i.e. the true land-cover proportion before the modal reduction.

| Code | LULC class | Cells | Area (km²) | Modal share | Fractional share |
| --- | --- | --- | --- | --- | --- |
| 1 | arable | 11,431,047 | 1,786.1 | 8.81% | 8.70% |
| 2 | permanent_crop | 3,314,701 | 517.9 | 2.56% | 2.50% |
| 3 | grassland | 24,175,069 | 3,777.4 | 18.64% | 18.32% |
| 4 | woody | 81,150,942 | 12,679.8 | 62.56% | 62.42% |
| 5 | built_up | 6,756,035 | 1,055.6 | 5.21% | 5.76% |
| 6 | water_wetland | 933,856 | 145.9 | 0.72% | 0.75% |
| 7 | bare_open | 1,959,509 | 306.2 | 1.51% | 1.52% |
|  | Total | 129,721,159 | 20,268.9 | 100.00% | 99.97% |

Total mapped area is 20,269 km² against Slovenia's official 20,271 km². Modal and fractional shares agree to within 0.6 percentage points for every class, confirming the modal reduction introduces no systematic bias.

6. Areal composition from the fractional bands

| LULC class | Share | Area (km²) |
| --- | --- | --- |
| woody | 62.42% | 12,655.6 |
| grassland | 18.32% | 3,714.0 |
| arable | 8.70% | 1,763.4 |
| built_up | 5.76% | 1,167.1 |
| permanent_crop | 2.50% | 507.7 |
| bare_open | 1.52% | 307.4 |
| water_wetland | 0.75% | 151.3 |
| Total | 99.97% | 20,266.5 |

Woody cover at 62.4% comprises forest (2000) together with the scrub and transitional classes 1410, 1420, 1500 and 1800; the forest class alone corresponds to the commonly cited national figure of roughly 58%.

7. Sub-cell purity

Distribution of the largest class fraction within each cell, quantifying what the modal reduction discards.

| Largest class fraction in cell | Cells | Share |
| --- | --- | --- |
| Pure (max fraction >= 0.99) | 108,351,297 | 83.51% |
| 0.90 - 0.99 | 5,073,932 | 3.91% |
| 0.60 - 0.90 | 11,885,206 | 9.16% |
| Mixed (< 0.60) | 4,439,350 | 3.42% |

83.5% of cells contain a single class, so the modal value is exact for the great majority of Slovenia. The remaining 16.5% are concentrated along land-use boundaries. Where that mixing matters, the fractional bands of raba_frac_202606.tif provide it directly.

8. Validation

| Check | Result |
| --- | --- |
| Grid identity with Flooded_NoFlood.tif | Pass - transform, dimensions and CRS identical |
| Sum of class fractions equals coverage band | Pass - max residual 6.333e-08 (float32 rounding) |
| Band value ranges | Pass - all fractions within [0, 1]; modal within [0, 7] |
| Unmapped RABA codes | Pass - none; all 25 codes assigned |
| Total area vs. official national area | Pass - 20,269 km² vs 20,271 km² (0.02% difference) |
| Valid cells in fraction raster | 129,749,785 |
| Valid cells in LULC.tif | 129,721,159 |
| Cells lost to RABA polygon gaps | 28,626 (0.022%) |

Polygon coverage per cell

| Coverage band value | Cells | Share |
| --- | --- | --- |
| >= 0.999 | 129,671,561 | 99.94% |
| 0.9 - 0.999 | 13,576 | 0.01% |
| 0.5 - 0.9 | 24,143 | 0.02% |
| < 0.5 | 40,505 | 0.03% |

Coverage is effectively complete on 99.94% of cells. The small remainder lies on the national border, where the RABA polygon coverage ends inside the raster mask.

9. Output files

| File | Description |
| --- | --- |
| For_Analysis/raba_frac_202606.tif | 9 bands, float32, 401 MB. Bands 1-7 are per-class areal fractions in [0, 1] in the order arable, permanent_crop, grassland, woody, built_up, water_wetland, bare_open; band 8 is polygon coverage; band 9 is the modal class id. Nodata NaN. |
| CgFs_Aligned/LULC.tif | 1 band, float32, LZW. Categorical conditioning factor, codes 1-7 as in Section 5. Nodata 0, denoting cells outside the Slovenia mask or without RABA polygon coverage. |
| For_Analysis/raba_fractions.py | Conversion script. Group mapping, oversampling factor and reference grid are command-line configurable. |

The nodata convention of LULC.tif matches the existing categorical factors (Hgk_grouped, Pedoloska_grouped, lito_poly_250_d96_grouped), which likewise use 0, so the raster is handled correctly by the existing sampling and training code without modification.